Title: Identification of ionic chloroacetanilide-herbicide metabolites in surface water and groundwater by HPLC/MS using negative ion spray

Author(s): Ferrer I, Thurman EM, Barcelo D

Source: ANALYTICAL CHEMISTRY 69 (22): 4547-4553 NOV 15 1997

Document Type: Article

Language: English

Cited References: 13      Times Cited: 57

Abstract: Solid-phase extraction (SPE) was combined with highperformance liquid chromatography/high-flow pneumatically assisted electrospray mass spectrometry (HPLC/ESP/MS) for the trace analysis of oxanilic and sulfonic acids of acetochlor, alachlor, and metolachlor. The isolation procedure separated the chloroacetanilide metabolites from the parent herbicides during the elution from C-18 cartridges using ethyl acetate for parent compounds, followed by methanol for the anionic metabolites. The metabolites were separated chromatographically using reversed-phase HPLC and analyzed by negative-ion MS using electrospray ionization in selected ion mode, Quantitation limits were 0.01 mu g/L for both the oxanilic and sulfonic acids based on a 100-mL water sample, This combination of methods represents an important advance in environmental analysis of chloroacetanilide-herbicide metabolites in surface water and groundwater for two reasons. First, anionic chloroacetanilide metabolites are a major class of degradation products that are readily leached to groundwater in agricultural areas. Second, anionic metabolites, which are not able to be analyzed by conventional methods such as liquid extraction and gas chromatography/mass spectrometry, are effectively analyzed by SPE and high-now pneumatically assisted electrospray mass spectrometry, This paper reports the first HPLC/MS identification of these metabolites in surface water and groundwater.

KeyWords Plus: MIDWESTERN UNITED-STATES; CHROMATOGRAPHY MASS-SPECTROMETRY; LINKED-IMMUNOSORBENT-ASSAY; ACID METABOLITE; ALACHLOR

Addresses: CSIC, CID, DEPT ENVIRONM CHEM, BARCELONA, ES-08034 SPAIN
US GEOL SURVEY, LAWRENCE, KS 66049 USA

Publisher: AMER CHEMICAL SOC, 1155 16TH ST, NW, WASHINGTON, DC 20036

Title: Status and analytical techniques of disinfection by-products in drinking water

Author(s): Dong LL, Huang JX

Source: PROGRESS IN CHEMISTRY 17 (2): 350-358 MAR 2005

Document Type: Article

Language: Chinese

Cited References: 73      Times Cited: 0

Abstract: The properties and status of disinfection by-products (DBPs) in drinking water produced by chlorine, ozone and chlorine dioxide are introduced. The analytical techniques of sample pretreatment for DBPs are generally reviewed. Future trends of development in this field are also proposed.

Author Keywords: drinking water; disinfectants; disinfection by-products; analytical techniques

KeyWords Plus: SOLID-PHASE MICROEXTRACTION; STRONG MUTAGEN 3-CHLORO-4-(DICHLOROMETHYL)-5-HYDROXY-2(5H)-FURANONE; ION-EXCLUSION CHROMATOGRAPHY; CATION-EXCHANGE RESIN; CHLORINATING MODEL COMPOUNDS; VOLATILE ORGANIC-COMPOUNDS; HALOACETIC ACIDS; MASS-SPECTROMETRY; GAS-CHROMATOGRAPHY; MX 3-CHLORO-4-(DICHLOROMETHYL)-5-HYDROXY-2(5H)-FURANONE

Addresses: Dong LL (reprint author), Chinese Acad Sci, Ecoenvironm Sci Res Ctr, Beijing, 100085 Peoples R China
Chinese Acad Sci, Ecoenvironm Sci Res Ctr, Beijing, 100085 Peoples R China

E-mail Addresses: junxionghuang@yahoo.com

Publisher: CHINESE ACAD SCIENCES, NO. 8 KEXUEYUANNANLU, ZHONGGUANCUN, BEIJING 100080, PEOPLES R CHINA

Subject Category: CHEMISTRY, MULTIDISCIPLINARY

IDS Number: 912JH

Title: Using ion chromatography to monitor haloacetic acids in drinking water: a review of current technologies

Author(s): Paull B, Barron L

Source: JOURNAL OF CHROMATOGRAPHY A 1046 (1-2): 1-9 AUG 13 2004

Document Type: Review

Language: English

Cited References: 37      Times Cited: 2

Abstract: A review of the application of ion chromatography to the determination of haloacetic acids in drinking water is given. As it requires no sample derivatisation, ion chromatography in its various modes, such as ion-exchange, ion-interaction and ion-exclusion chromatography, is increasingly being investigated as a simpler alternative to gas chromatographic methods for the determination of polar disinfection by-products (DBPs) in drinking waters. Detection limits quoted for the regulated haloacetic acids (HAA5), are commonly in the mid to low mug/L range, however, in most cases analyte preconcentration is still necessary for detection at concentrations commonly found in actual drinking water samples. The coupling of ion chromatography to electrospray mass spectrometry provides a potential future direction, with improved sensitivity and selectivity compared to conductivity based detection, however associated cost and complexity for routine analysis is currently relatively high. (C) 2004 Elsevier B.V. All rights reserved.

Author Keywords: reviews; water analysis; haloacetic acids

KeyWords Plus: CATION-EXCHANGE RESIN; MASS-SPECTROMETRIC DETECTION; DISINFECTION BY-PRODUCTS; EXCLUSION CHROMATOGRAPHY; CARBOXYLIC-ACIDS; CHLORINATION; TRIHALOMETHANES; HAAS; THMS

Addresses: Paull B (reprint author), Dublin City Univ, Natl Ctr Sensor Res, Sch Chem Sci, Dublin, 9 Ireland
Dublin City Univ, Natl Ctr Sensor Res, Sch Chem Sci, Dublin, 9 Ireland

E-mail Addresses: brett.paull@dcu.ie

Publisher: ELSEVIER SCIENCE BV, PO BOX 211, 1000 AE AMSTERDAM, NETHERLANDS

Subject Category: CHEMISTRY, ANALYTICAL; BIOCHEMICAL RESEARCH METHODS

IDS Number: 847AP

Title: Characterizing dissolved organic carbon using asymmetrical flow field-flow fractionation with on-line UV and DOC detection

Author(s): Reszat TN, Hendry MJ

Source: ANALYTICAL CHEMISTRY 77 (13): 4194-4200 JUL 1 2005

Document Type: Article

Language: English

Cited References: 41      Times Cited: 0

Abstract: A method of characterizing dissolved organic carbon (DOC) by asymmetrical flow field-flow fractionation with on-line UV and DOC detection is described and applied to standards and natural water samples. Poly(styrenesulfonate) polymer standards, Suwannee River humic standards, and naturally occurring surface water and groundwater DOC were analyzed using this coupled detection technique. Molecular weight determinations in the samples and standards were 6-30% lower with DOC analysis than UV analysis. This difference was attributed to the insensitivity of the latter technique to nonaromatic carbon and suggests the molecular weight determined with the DOC detector is a more accurate representation of the actual molecular weight of the DOC. A normalized intensity comparison (NIC) method was proposed to distinguish differences in the relative amounts of aromatic and aliphatic carbon in DOC by comparing the two detector responses. The NIC method was applied to yield an average aromatic content of the bulk DOC and to detail the aromatic content over a range of molecular weights in a single DOC fraction.

KeyWords Plus: MOLECULAR-WEIGHT DISTRIBUTIONS; SIZE-EXCLUSION CHROMATOGRAPHY; CLAY-RICH AQUITARD; HUMIC SUBSTANCES; FULVIC-ACIDS; PORE WATERS; MATTER; OPTIMIZATION; COLLOIDS; AQUIFER

Addresses: Reszat TN (reprint author), Univ Saskatchewan, Dept Geol Sci, Saskatoon, SK S7N 5E2 Canada
Univ Saskatchewan, Dept Geol Sci, Saskatoon, SK S7N 5E2 Canada

E-mail Addresses: thr690@mail.usask.ca

Publisher: AMER CHEMICAL SOC, 1155 16TH ST, NW, WASHINGTON, DC 20036 USA

Subject Category: CHEMISTRY, ANALYTICAL

Title: Review of analytical methods for the determination of nonylphenol and related compounds in environmental samples

Author(s): Lee HB

Source: WATER QUALITY RESEARCH JOURNAL OF CANADA 34 (1): 3-35 1999

Document Type: Review

Language: English

Cited References: 97      Times Cited: 41

Abstract: Analytical methods published in the last 20 years for the extraction, chromatographic separation, and quantification of alkylphenol ethoxylates (APEO) and related compounds in environmental samples are reviewed. Examples of various isolation and preconcentration techniques for water, effluent, sediment and sludge are presented. This includes procedures from the classical liquid-liquid and Soxhlet extraction to the up-to-date solid phase and supercritical fluid extraction. Chromatographic separation of APEO by normal and reversed phase liquid chromatography (LC) and capillary column gas chromatography (GC) is compared. A variety of quantification methods involving the common LC and GC detectors as well as various mass spectrometric techniques are also discussed.

Author Keywords: nonylphenol ethoxylates; nonylphenol; octylphenol; determination; environmental samples

KeyWords Plus: PERFORMANCE LIQUID-CHROMATOGRAPHY; ALKYLPHENOL POLYETHOXYLATE SURFACTANTS; PERSISTENT ORGANIC-CHEMICALS; CAPILLARY GAS-CHROMATOGRAPHY; SUPERCRITICAL-FLUID CHROMATOGRAPHY; MASS-SPECTROMETRIC ANALYSIS; SOLID-PHASE EXTRACTION; NON-IONIC SURFACTANTS; NONIONIC SURFACTANTS; LINEAR ALKYLBENZENESULFONATES

Addresses: Lee HB (reprint author), Environm Canada, Canada Ctr Inland Waters, Aquat Ecosyst Protect Branch, 867 Lakeshore Rd, Burlington, ON L7R 4A6 Canada
Environm Canada, Canada Ctr Inland Waters, Aquat Ecosyst Protect Branch, Burlington, ON L7R 4A6 Canada

Publisher: CANADIAN ASSOC WATER QUALITY, C/O DR H R EISENHAUER, ENVIRONMENTAL TECHNOL CENTRE, 3439 RIVER ROAD SOUTH, GLOUCESTER, ONTARIO K1A 0H3, CANADA

Subject Category: WATER RESOURCES

Title: Identification of organic residues in tertiary effluents by GC/EI-MS, GC/CI-MS and GC/TSQ-MS

Author(s): Ding WH, Fujita Y, Aeschimann R, Reinhard M

Source: FRESENIUS JOURNAL OF ANALYTICAL CHEMISTRY 354 (1): 48-55 JAN 1996

Document Type: Article

Language: English

Cited References: 24      Times Cited: 30

Abstract: A method was developed for the analysis of non-volatile dissolved organic residues in tertiary treated wastewater effluents. This method involved concentration of samples by rotary evaporation, propylation using propanol/formic acid/acetyl chloride, and separation, detection and quantitation by capillary GC/EI and CI-MS and GC/TSQ-MS analysis. Ethylenediamine tetraacetic acid (EDTA) was the most prominent compound found in both granular activated carbon (GAG) and chlorinated GAC effluents (110 and 140 mu g/L, respectively). Other compounds identified included nitrilotriacetic acid (NTA), carboxyalkylphenoxy ethoxy carboxylates, poly(propoxy), poly(ethoxy) or poly(ethoxy)(propoxy) compounds, small aliphatic dicarboxylic acids and aldehydes, all at mu g/L levels. Approximately 80% of all chromatographically separated compounds were positively or tentatively identified. The identified compounds are estimated to account for approximately 10% of the dissolved organic carbon.

KeyWords Plus: CHROMATOGRAPHY MASS-SPECTROMETRY; RESOLUTION GAS-CHROMATOGRAPHY; ACID; WATER

Addresses: Ding WH (reprint author), STANFORD UNIV, DEPT CIVIL ENGN, STANFORD, CA 94305 USA
ETH ZURICH, ORGAN CHEM ABT, ZURICH, SWITZERLAND

Publisher: SPRINGER VERLAG, 175 FIFTH AVE, NEW YORK, NY 10010

Subject Category: CHEMISTRY, ANALYTICAL

IDS Number: TN471

Title: Analysis of nonylphenol polyethoxylates and their degradation products in river water and sewage effluent by gas chromatography ion trap (tandem) mass spectrometry with electron impact and chemical ionization

Author(s): Ding WH, Tzing SH

Source: JOURNAL OF CHROMATOGRAPHY A 824 (1): 79-90 OCT 16 1998

Document Type: Article

Language: English

Cited References: 26      Times Cited: 44

Abstract: A method is presented for the analysis of nonylphenol polyethoxylate (NPEO) residues and their degradation products, nonylphenol polyethoxy carboxylates and carboxyalkylphenol ethoxy carboxylates, in the samples of river water and sewage effluent. The method involves extraction of the samples by graphitized carbon black (GCB) cartridge, propylation by a propanol/acetyl chloride derivatization procedure, and separation, identification and quantitation by ion-trap GC-MS with electron impact ionization (EI), liquid-chemical ionization (CI) and CI-MS-MS modes. The large-volume injection technique provides high precision and sensitivity for both NPEO residues and their degradation products, to quantitation at greater than or equal to 0.01 mu g/l in 100 ml of water samples. Dicarboxylic acids of NPEO residues were identified by the CI-MS-MS technique with relatively high concentrations in the samples of river water and sewage effluent. Recovery of nonylphenol and octylphenoxyacetic acid in spiked water samples ranged from 81 to 107%. Relative standard deviations of replicate analyses ranged from 2 to 12%. (C) 1998 Elsevier Science B.V. All rights reserved.

Author Keywords: water analysis; sewage effluent; environmental analysis; nonylphenol polyethoxylates; polyethoxylates; surfactants

KeyWords Plus: NONIONIC SURFACTANTS; AQUATIC ENVIRONMENT; ALKYLPHENOL; TRANSFORMATION; IDENTIFICATION; CARBOXYLATES; BEHAVIOR; RESIDUES; SPECTRA; MS

Addresses: Ding WH (reprint author), Natl Cent Univ, Dept Chem, Chungli, 32054 Taiwan
Natl Cent Univ, Dept Chem, Chungli, 32054 Taiwan

Publisher: ELSEVIER SCIENCE BV, PO BOX 211, 1000 AE AMSTERDAM, NETHERLANDS

Subject Category: CHEMISTRY, ANALYTICAL; BIOCHEMICAL RESEARCH METHODS

IDS Number: 133PM

Title: Identification of metabolites from the biological transformation of the nonionic surfactant residue octylphenoxyacetic acid and its brominated analog

Author(s): Fujita Y, Reinhard M

Source: ENVIRONMENTAL SCIENCE & TECHNOLOGY 31 (5): 1518-1524 MAY 1997

Document Type: Article

Language: English

Cited References: 44      Times Cited: 22

Abstract: The aerobic biological transformation of octylphenoxyacetic acid (OP1EC) and its brominated analog (BrOP1EC) by groundwater enrichment cultures was studied, and persistent metabolites were identified by GC/MS. OP1EC is a representative of the class of alkylphenol ethoxycarboxylates (APEC), formed from alkylphenol polyethoxylate nonionic surfactants during sewage treatment. BrOP1EC is a byproduct formed during chlorine disinfection in the presence of bromide. The metabolite 2,4,4-trimethyl-2-pentanol was detected in stoichiometric quantities in OP1EC-metabolizing enrichment cultures, representing the intact alkyl side chain as a tertiary alcohol. BrOP1EC was transformed by the OP1EC-utilizing cultures only if OP1EC was simultaneously metabolized, suggesting a cometabolic mechanism of transformation. Brominated intermediates were also detected: brominated octylphenol and a compound tentatively identified as 2-aminomethoxy-3-bromo-5-(1,1,3,3-tetramethylbutyl)phenol.

KeyWords Plus: ALKYLPHENOL POLYETHOXYLATE SURFACTANTS; LIQUID-CHROMATOGRAPHY; MASS-SPECTROMETRY; AQUATIC ENVIRONMENT; SEWAGE-TREATMENT; TREATMENT PLANT; NONYL-PHENOL; ETHOXYLATES; WATER; BIODEGRADABILITY

Addresses: STANFORD UNIV, DEPT CIVIL ENGN, STANFORD, CA 94305 USA

Publisher: AMER CHEMICAL SOC, 1155 16TH ST, NW, WASHINGTON, DC 20036

Subject Category: ENGINEERING, ENVIRONMENTAL; ENVIRONMENTAL SCIENCES

IDS Number: WW948

ISSN: 0013-936X

The below papers are examples of PCR methods applied to detect Viruses in Environmental Waters

Title: Sensor components for PCR detection of Salmonella in alfalfa sprout irrigation water

Author(s): Jackson E, Delwiche M, Barak J, Charkowski A, Suslow T

Source: TRANSACTIONS OF THE ASAE 47 (6): 2137-2144 NOV-DEC 2004

Document Type: Article

Language: English

Cited References: 22      Times Cited: 0

Abstract: Salmonella has been associated with a number of recent outbreaks of foodborne illness involving sprouted seeds. In this research, we addressed the design of sensor components intended for use in a fully automated detection system. A real-time PCR assay was developed to detect Salmonella in sprout irrigation water. Specific detection was achieved by targeting a region of the invasion gene, invA. The detection limit for Salmonella in sterile water was approximately 400 CFU, and in sprout irrigation water the detection limit was approximately 200 CFU. Components for an automated sensing system were designed, including a thermal cycler and a fluorescent optical sensor The thermal cycler utilized a thermoelectric module for heating and cooling the sample block and a heat sink and fan to remove heat from the module during cooling. Sample temperature was controlled to within about 1degreesC at each of the PCR setpoints (94degreesC, 55degreesC, and 72degreesC). The optical sensor used a laser diode (635 nm) for excitation and a bandpass interference filter (700 +/- 20 nm) coupled with a photodiode for fluorescence detection. The DNA dye TO-PRO-3 (642 nm excitation; 661 nm emission) was chosen to match the excitation wavelength of the laser diode. Calibration of the optical sensor with calf thymus DNA showed detection down to 0.01 mug mL(-1), demonstrating the potential to detect down to 1 CFU if used in conjunction with PCR. When the sensor components were used to implement the real-time assay, S. Newport was detected to approximately 7.3 x 10(4) CFU in sterile water and approximately 1.5 x 10(6) CFU in sprout irrigation water. Further optimization of the assay in the sensor will be needed to improve the detection limits.

Author Keywords: biosensor food safety; pathogens; real-time PCR

KeyWords Plus: POLYMERASE-CHAIN-REACTION; ESCHERICHIA-COLI O157-H7; BIOSENSOR; BACTERIA; AMPLIFICATION; SAMPLES; ASSAY

Addresses: Delwiche M (reprint author), Univ Calif Davis, Dept Biol & Agr Engn, 1 Shield Ave, Davis, CA 95616 USA
Univ Calif Davis, Dept Biol & Agr Engn, Davis, CA 95616 USA
USDA ARS, Western Reg Res Ctr, Albany, CA 94710 USA
Univ Wisconsin, Dept Plant Pathol, Madison, WI 53706 USA
Univ Calif Davis, Dept Veg Crops, Davis, CA 95616 USA

E-mail Addresses: mjdelwiche@ucdavis.edu

Publisher: AMER SOC AGRICULTURAL ENGINEERS, 2950 NILES RD, ST JOSEPH, MI 49085-9659 USA

Subject Category: AGRICULTURAL ENGINEERING

PDF File of Paper Found?

Review Notes

Title: An efficient virus concentration method and RT-nested PCR for detection of rotaviruses in environmental water samples

Author(s): Kittigul L, Ekchaloemkiet S, Utrarachkij F, Siripanichgon K, Sujirarat D, Pungchitton S, Boonthum A

Source: JOURNAL OF VIROLOGICAL METHODS 124 (1-2): 117-122 MAR 2005

Document Type: Article

Language: English

Cited References: 27      Times Cited: 0

Abstract: Water samples were concentrated by the modified adsorption-elution technique followed by speedVac reconcentration of the filter eluates. Reverse transcriptase-nested polymerase chain reaction (RT-nested PCR) was used to detect rotavirus RNA in concentrates of the water. The detection limit of the rotavirus determined by RT-nested PCR alone was about 1.67 plaque forming units (PFU) per RT-PCR assay and that by RT-nested PCR combined with concentration from I I seeded tap water sample was 1.46 plaque forming units per assay. Water samples were collected from various sources, concentrated, and determined rotavirus RNA. Of 120 water samples, rotavirus RNA was detected in 20 samples (16.7%); 2/10 (20%) of the river samples, 8/30 (26.7%) of the canal samples, and 10/40 (25%) of the sewage samples but was not found in any tap water samples (0/40). Only three water samples were positive for rotavirus antigen determined using an enzyme-linked immunosorbent assay (ELISA). Alignment analysis of the sequenced PCR product (346-bp fragment) was performed in eight rotavirus-positive samples using the rotavirus sequence deposited in the GenBank. All samples gave the correct VP7 sequence. Results of analysis showed two samples similar to human rotavirus (97-98%), five similar to rotavirus G9 sequence (94-99%), and one sample similar to animal rotavirus (97%). PCR inhibitors were not observed in any concentrated water samples. In all 20 (of 120) samples where rotaviruses were found, fecal coliforms including Escherichia coli were also found, but of the samples testing negative for rotaviruses, 76 were fecal coliforms positive and 69 were E. coli positive. The combination of the virus concentration method and RT-nested PCR described below made it possible to effectively detect rotaviruses in environmental water samples. (C) 2004 Elsevier B.V. All rights reserved.

Author Keywords: virus concentration method; RT-nested PCR; rotaviruses; water samples

KeyWords Plus: POLYMERASE-CHAIN-REACTION; REVERSE TRANSCRIPTION; DRINKING-WATER; SEWAGE; ENTEROVIRUSES; GROUNDWATER; SURVIVAL; GASTROENTERITIS; INHIBITORS; OUTBREAK

Addresses: Kittigul L (reprint author), Mahidol Univ, Fac Publ Hlth, Dept Microbiol, 420-1 Rajvithi Rd, Bangkok, 10400 Thailand
Mahidol Univ, Fac Publ Hlth, Dept Microbiol, Bangkok, 10400 Thailand
Mahidol Univ, Fac Publ Hlth, Dept Epidemiol, Bangkok, 10400 Thailand
Mahidol Univ, Fac Publ Hlth, Rural Hlth Training & Res Ctr, Bangkok, 10400 Thailand

E-mail Addresses: phlkt@mahidol.ac.th

PDF File of Paper Found?

Review Notes

Title: A strategy for detection of viruses in groundwater by PCR

Author(s): Abbaszadegan M, Stewart P, LeChevallier M

Source: APPLIED AND ENVIRONMENTAL MICROBIOLOGY 65 (2): 444-449 FEB 1999

Document Type: Article

Language: English Cited References: 15      Times Cited: 42

Abstract: We evaluated the use of the PCR for detection of enteric viruses in groundwater. To do this, we used an improved sample-processing technique and a large-volume amplification protocol. The objective of this study was to use advanced molecular techniques to develop a rapid and simple method which can be used by the water industry for detection of viral contamination in a variety of water samples. The strategy described here fulfills the water industry's need for a rapid, reliable, easily performed method for analyzing groundwater for virus contamination. Viruses were detected after concentration from at least 400 gallons (1,512 liters) of water by a filter adsorption and elution method, which resulted in a concentrate containing viruses. A total of 150 samples were analyzed by performing cell culture assays for enteroviruses and by performing reverse transcription PCR (RT-PCR) analyses for enteroviruses, hepatitis A virus, and rotavirus. Thirteen samples (8.7%) produced cellular cytopathic effects when the Buffalo green monkey cell line was used. When primers specific for enteroviruses were used in RT-PCR, 40 of 133 samples (30.1%) tested positive for the presence of enterovirus RNA. When hepatitis A virus-specific primers were used, 12 of 139 samples (8.6%) were considered positive for the presence of hepatitis A viral RNA. The RT-PCR analysis performed with rotavirus-specific primers identified 18 of 130 samples (13.8%) that were positive for rotavirus RNA sequences. Our sample-procesing technique and large-volume PCR protocol (reaction volume, 300 mu l) resulted in sufficient removal or dilution of inhibitors so that more than 95% of the samples could be assayed by PCR, Because of its sensitivity for detecting viral nucleic acid sequences, PCR analysis should produce more positive results than cell culture analysis. Since either cell culture analysis or PCR can reveal only a ''snapshot" of the quality of the groundwater being sampled, PCR seems to be a desirable rapid initial screening tool.

KeyWords Plus: POLYMERASE CHAIN-REACTION; ENTERIC VIRUSES; DRINKING-WATER; DNA; ENTEROVIRUSES; AMPLIFICATION

Addresses: Abbaszadegan M (reprint author), Amer Water Works Serv Co Inc, Qual Control & Res Lab, 1115 S Illinois St, Belleville, IL 62220 USA
Amer Water Works Serv Co Inc, Qual Control & Res Lab, Belleville, IL 62220 USA
Amer Water Works Co Inc, Voorhees, NJ 08043 USA

Publisher: AMER SOC MICROBIOLOGY, 1325 MASSACHUSETTS AVENUE, NW, WASHINGTON, DC 20005-4171 USA

Subject Category: BIOTECHNOLOGY & APPLIED MICROBIOLOGY; MICROBIOLOGY

IDS Number: 165AC

PDF File of Paper Found?

Review Notes

Title: An efficient virus concentration method and RT-nested PCR for detection of rotaviruses in environmental water samples

Author(s): Kittigul L, Ekchaloemkiet S, Utrarachkij F, Siripanichgon K, Sujirarat D, Pungchitton S, Boonthum A

Source: JOURNAL OF VIROLOGICAL METHODS 124 (1-2): 117-122 MAR 2005

Document Type: Article

Language: English

Cited References: 27      Times Cited: 0

Abstract: Water samples were concentrated by the modified adsorption-elution technique followed by speedVac reconcentration of the filter eluates. Reverse transcriptase-nested polymerase chain reaction (RT-nested PCR) was used to detect rotavirus RNA in concentrates of the water. The detection limit of the rotavirus determined by RT-nested PCR alone was about 1.67 plaque forming units (PFU) per RT-PCR assay and that by RT-nested PCR combined with concentration from I I seeded tap water sample was 1.46 plaque forming units per assay. Water samples were collected from various sources, concentrated, and determined rotavirus RNA. Of 120 water samples, rotavirus RNA was detected in 20 samples (16.7%); 2/10 (20%) of the river samples, 8/30 (26.7%) of the canal samples, and 10/40 (25%) of the sewage samples but was not found in any tap water samples (0/40). Only three water samples were positive for rotavirus antigen determined using an enzyme-linked immunosorbent assay (ELISA). Alignment analysis of the sequenced PCR product (346-bp fragment) was performed in eight rotavirus-positive samples using the rotavirus sequence deposited in the GenBank. All samples gave the correct VP7 sequence. Results of analysis showed two samples similar to human rotavirus (97-98%), five similar to rotavirus G9 sequence (94-99%), and one sample similar to animal rotavirus (97%). PCR inhibitors were not observed in any concentrated water samples. In all 20 (of 120) samples where rotaviruses were found, fecal coliforms including Escherichia coli were also found, but of the samples testing negative for rotaviruses, 76 were fecal coliforms positive and 69 were E. coli positive. The combination of the virus concentration method and RT-nested PCR described below made it possible to effectively detect rotaviruses in environmental water samples. (C) 2004 Elsevier B.V. All rights reserved.

Author Keywords: virus concentration method; RT-nested PCR; rotaviruses; water samples

KeyWords Plus: POLYMERASE-CHAIN-REACTION; REVERSE TRANSCRIPTION; DRINKING-WATER; SEWAGE; ENTEROVIRUSES; GROUNDWATER; SURVIVAL; GASTROENTERITIS; INHIBITORS; OUTBREAK

Addresses: Kittigul L (reprint author), Mahidol Univ, Fac Publ Hlth, Dept Microbiol, 420-1 Rajvithi Rd, Bangkok, 10400 Thailand
Mahidol Univ, Fac Publ Hlth, Dept Microbiol, Bangkok, 10400 Thailand
Mahidol Univ, Fac Publ Hlth, Dept Epidemiol, Bangkok, 10400 Thailand
Mahidol Univ, Fac Publ Hlth, Rural Hlth Training & Res Ctr, Bangkok, 10400 Thailand

E-mail Addresses: phlkt@mahidol.ac.th

PDF File of Paper Found?

Review Notes

Title: Sensor components for PCR detection of Salmonella in alfalfa sprout irrigation water

Author(s): Jackson E, Delwiche M, Barak J, Charkowski A, Suslow T

Source: TRANSACTIONS OF THE ASAE 47 (6): 2137-2144 NOV-DEC 2004

Document Type: Article

Language: English

Cited References: 22      Times Cited: 0

Abstract: Salmonella has been associated with a number of recent outbreaks of foodborne illness involving sprouted seeds. In this research, we addressed the design of sensor components intended for use in a fully automated detection system. A real-time PCR assay was developed to detect Salmonella in sprout irrigation water. Specific detection was achieved by targeting a region of the invasion gene, invA. The detection limit for Salmonella in sterile water was approximately 400 CFU, and in sprout irrigation water the detection limit was approximately 200 CFU. Components for an automated sensing system were designed, including a thermal cycler and a fluorescent optical sensor The thermal cycler utilized a thermoelectric module for heating and cooling the sample block and a heat sink and fan to remove heat from the module during cooling. Sample temperature was controlled to within about 1degreesC at each of the PCR setpoints (94degreesC, 55degreesC, and 72degreesC). The optical sensor used a laser diode (635 nm) for excitation and a bandpass interference filter (700 +/- 20 nm) coupled with a photodiode for fluorescence detection. The DNA dye TO-PRO-3 (642 nm excitation; 661 nm emission) was chosen to match the excitation wavelength of the laser diode. Calibration of the optical sensor with calf thymus DNA showed detection down to 0.01 mug mL(-1), demonstrating the potential to detect down to 1 CFU if used in conjunction with PCR. When the sensor components were used to implement the real-time assay, S. Newport was detected to approximately 7.3 x 10(4) CFU in sterile water and approximately 1.5 x 10(6) CFU in sprout irrigation water. Further optimization of the assay in the sensor will be needed to improve the detection limits.

Author Keywords: biosensor food safety; pathogens; real-time PCR

KeyWords Plus: POLYMERASE-CHAIN-REACTION; ESCHERICHIA-COLI O157-H7; BIOSENSOR; BACTERIA; AMPLIFICATION; SAMPLES; ASSAY

Addresses: Delwiche M (reprint author), Univ Calif Davis, Dept Biol & Agr Engn, 1 Shield Ave, Davis, CA 95616 USA
Univ Calif Davis, Dept Biol & Agr Engn, Davis, CA 95616 USA
USDA ARS, Western Reg Res Ctr, Albany, CA 94710 USA
Univ Wisconsin, Dept Plant Pathol, Madison, WI 53706 USA
Univ Calif Davis, Dept Veg Crops, Davis, CA 95616 USA

E-mail Addresses: mjdelwiche@ucdavis.edu

Publisher: AMER SOC AGRICULTURAL ENGINEERS, 2950 NILES RD, ST JOSEPH, MI 49085-9659 USA

Subject Category: AGRICULTURAL ENGINEERING

PDF File of Paper Found?

Review Notes

Title: A multiplex reverse transcription-PCR method for detection of human enteric viruses in groundwater

Author(s): Fout GS, Martinson BC, Moyer MWN, Dahling DR

Source: APPLIED AND ENVIRONMENTAL MICROBIOLOGY 69 (6): 3158-3164 JUN 2003

Document Type: Article

Language: English

Cited References: 30      Times Cited: 15

Abstract: Untreated groundwater is responsible for about half of the waterborne disease outbreaks in the United States. Human enteric viruses are thought to be leading etiological agents of many of these outbreaks, but there is relatively little information on the types and levels of viruses found in groundwater. To address this problem, monthly samples from 29 groundwater sites were analyzed for I year for enteroviruses, hepatitis A virus, Norwalk virus, reoviruses, and rotaviruses by multiplex reverse transcription-PCR (RT-PCR). A procedure with which to remove environmental RT-PCR inhibitors from groundwater samples was developed. The procedure allowed an average of 71 liters of the original groundwater to be assayed per RT-PCR, with an average virus recovery rate of 74%, based on seeded samples. Human enteric viruses were detected in 16% of the groundwater samples analyzed, with reoviruses being the most frequently detected virus group.

KeyWords Plus: POLYMERASE CHAIN-REACTION; VIRAL GASTROENTERITIS; WATERBORNE OUTBREAK; UNITED-STATES; NORWALK VIRUS; WELL WATER; ENTEROVIRUSES; CONTAMINATION; INHIBITORS

Addresses: Fout GS (reprint author), US EPA, NERL, Off Res & Dev, Martin Luther King Dr, Cincinnati, OH 45268 USA
US EPA, NERL, Off Res & Dev, Cincinnati, OH 45268 USA
US EPA, Tech Support Ctr, Off Water, Cincinnati, OH 45268 USA

Publisher: AMER SOC MICROBIOLOGY, 1752 N ST NW, WASHINGTON, DC 20036-2904 USA

Subject Category: BIOTECHNOLOGY & APPLIED MICROBIOLOGY; MICROBIOLOGY

PDF File of Paper Found?

Review Notes

Title: Deviation from the classical colloid filtration theory in the presence of repulsive DLVO interactions

Author(s): Tufenkji N, Elimelech M

Source: LANGMUIR 20 (25): 10818-10828 DEC 7 2004

Document Type: Article

Language: English

Cited References: 58      Times Cited: 4

Abstract: A growing body of experimental evidence suggests that the deposition behavior of microbial particles (e.g., bacteria and viruses) is inconsistent with the classical colloid filtration theory (CFT). Well-controlled laboratory-scale column deposition experiments were conducted with uniform model particles and collectors to obtain insight into the mechanisms that give rise to the diverging deposition behavior of microorganisms. Both the fluid-phase effluent particle concentration and the profile of retained particles were systematically measured over a broad range of physicochemical conditions. The results indicate that, in the presence of repulsive Derjaguin-Landau-Verwey-Overbeek (DLVO) interactions, the concurrent existence of both favorable and unfavorable colloidal interactions causes significant deviation from the CFT. A dual deposition mode model is presented which considers the combined influence of "fast" and "slow" particle deposition. This model is shown to adequately describe both the spatial distribution of particles in the packed bed and the suspended particle concentration at the column effluent.

KeyWords Plus: SATURATED POROUS-MEDIA; PARTICLE DEPOSITION; SURFACE INTERACTIONS; BROWNIAN PARTICLES; COLUMN EXPERIMENTS; BED FILTRATION; TRANSPORT; KINETICS; BACTERIA; RATES

Addresses: Tufenkji N (reprint author), McGill Univ, Dept Chem Engn, 3640 Univ St, Montreal, PQ H3A 2B2 Canada
McGill Univ, Dept Chem Engn, Montreal, PQ H3A 2B2 Canada
Yale Univ, Dept Chem Engn, Environm Engn Program, New Haven, CT 06520 USA

E-mail Addresses: nathalie.tufenkji@mcgill.ca

PDF File of Paper Found?

Review Notes