- BASIC SEISMIC PROCESSING USING LINUX, SUNIX AND MATLAB

CONTENTS

Pre-stack Processing

- Download field data

- Cut the data down to 1 s trace lengths

- Resample data to have a 0.5 ms (500 microseconds) sample interval

- Kill bad traces

- Add geometry to the trace headers

- Create CMP in trace header

- NMO (Normal Moveout) Correction

- Stack

Download field data  	Pre-stack TOC

Start by bringing a copy of the data set we collected on 10-11-05, from odyssey:

cp /home/refseis05/102205.tar /home/your_home_directory

After expanding the tarred file, you should have 38 files named like the following:

1001.su, 1002.su, 1003.su etc.  All files have a .su extension to distinguish that they are S*nix formatted files.  The original field data was collected in a different format and had to be converted.  Take the time to look at the contents of these files.  Find out the following information:

sample rate

number of samples

length of data in time

time of first sample

time of last sample

Nyquist frequency for the sample rate

Cut the data down to ~1 s trace lengths	Pre-stack TOC

Use sukill and suwind

Resample data to have a 0.5 ms (500 microseconds) sample interval	Pre-stack TOC

Kill bad traces		Pre-stack TOC

For an example of removing bad traces see kl_cut.sh

Add geometry to the trace headers	Pre-stack TOC

For an example of removing bad traces see header_geometry.sh

Create CMP in trace header	Pre-stack TOC

Use suchw

On odyssey: /home/refseis05/shell_exs/makecmp.sh

For an example of creating CMP values in headers see makecmp.sh

Sort data into CMP gathers	Pre-stack TOC

Use susort

On odyssey: /home/refseis05/shell_exs/study_NMO_STACK.sh

For an example of creating CMP values in headers see study_NMO_STACK.sh

NMO (Normal Moveout) Correction    		Pre-stack TOC

Use sunmo

On odyssey: /home/refseis05/shell_exs/study_NMO_STACK.sh

For an example of normal moveout see study_NMO_STACK.sh

On odyssey: /home/refseis05/shell_exs/makecmp.sh

See example within this document see study_NMO_STACK.sh

Stack		Pre-stack TOC

Use sustack

On odyssey: /home/refseis05/shell_exs/study_NMO_STACK.sh

See example within this document: study_NMO_STACK.sh

kl_cut.sh:    	Kill bad Traces	Pre-stack TOC

Purpose: delete bad traces and shorten data

#! /bin/sh

set -x

# DATA DIRECTORY

data='/data2/gom/Livingston_raw/seismics/data/SU/102205'

suximage_par='wbox=250 hbox=700'

#echo $data

file1='1001'

# VIEW GAINED DATA BEFORE TRACE KILL

sugain 	<$data/$file1.su 			\

agc=1 wagc=0.1  			\

|	suximage 					\

$suximage_par		 		\

xbox=50 				\

clip=5 title='RAW'&

# VIEW FILTERED DATA BEFORE TRACE KILL

sufilter <$data/$file1.su				\

f=3,6,120,250				\

|	sugain 						\

agc=1 wagc=0.1  			\

|	suximage 					\

$suximage_par	 			\

xbox=300 				\

clip=10 title='f=3,6,120,250 AGC .1'&

# VIEW DATA AFTER TRACE KILL

sukill <$data/$file1.su 				\

min=12 count=1 				\

| 	sukill 						\

min=15 count=1 				\

| 	sukill 						\

min=18 count=1 				\

| 	sukill 						\

min=23 count=1 				\

|       sukill                                          \

min=24 count=1                          \

| 	suwind 						\

tmin=0 tmax=1.0		 		\

|	sugain  					\

agc=1  wagc=0.1				\

| 	suximage 					\

$suximage_par				\

xbox=550 				\

clip=10 title='CUT KIL'&

# VIEW FILTERED DATA AFTER TRACE KILL

sukill <$data/$file1.su                           \

min=12 count=1                          \

|       sukill                                          \

min=15 count=1                          \

|       sukill                                          \

min=18 count=1                          \

|       sukill                                          \

min=23 count=1                          \

|       sukill                                          \

min=24 count=1                          \

|       suwind                                          \

tmin=0 tmax=1.0                         \

|	sufilter 					\

f=3,6,120,250				\

|	sugain 						\

agc=1 wagc=0.1  			\

|	suximage 					\

$suximage_par				\

xbox=800				\

clip=10 title='CUT KIL f=3,6,120,250'&

header_geom.sh    	Create offset,sx,gx,tracf values in headers	Pre-stack TOC

Purpose: delete bad traces and shorten data

#! /bin/sh

set -x

#Purpose: Generate appropriate headers before generating CMPs (see make_CMP.sh)

offset is defined as sx-gx

# DATA/$file DIRECTORY

DATA=/data2/gom/Livingston_raw/seismics/data/SU/102205

# CONCATENATE all the files

file1='1001.su'

file2='1002.su'

file3='1003.su'

file4='1004.su'

file5='1005.su'

file6='1006.su'

file7='1007.su'

file8='1008.su'

file9='1009.su'

file10='1010.su'

file11='1011.su'

file22='1012.su'

file13='1013.su'

file14='1014.su'

file15='1015.su'

file16='1016.su'

file17='1017.su'

file18='1018.su'

file19='1019.su'

file20='1020.su'

file21='1021.su'

file22='1022.su'

file23='1023.su'

file24='1024.su'

file25='1025.su'

file26='1026.su'

file27='1027.su'

file28='1028.su'

file29='1029.su'

file30='1030.su'

file31='1031.su'

file32='1032.su'

file33='1033.su'

file34='1034.su'

file35='1035.su'

file36='1036.su'

file37='1037.su'

file38='1038.su'

cat       $DATA/$file1  $DATA/$file2  $DATA/$file3  $DATA/$file4  $DATA/$file5  $DAT

A/$file6  $DATA/$file7  $DATA/$file8  $DATA/$file9 \

$DATA/$file10 $DATA/$file11 $DATA/$file12 $DATA/$file13 $DATA/$file14 $DATA/$file15

$DATA/$file16 $DATA/$file17 $DATA/$file18 $DATA/$file19 \

$DATA/$file20 $DATA/$file21 $DATA/$file22 $DATA/$file23 $DATA/$file24 $DATA/$file25

$DATA/$file26 $DATA/$file27 $DATA/$file28 $DATA/$file29 \

$DATA/$file30 $DATA/$file31 $DATA/$file32 $DATA/$file33 $DATA/$file34 $DATA/$file35

$DATA/$file36 $DATA/$file37 $DATA/$file38       \

| sushw                                 \

key=sx,offset,gx,fldr,tracf     \

a=0,33,-33,1001,1               \

b=0,3,-3,0,1                    \

c=3,0,3,0,0                     \

j=24,24,24,24,24                \

>$DATA/1001_38_geom.su

makecmp.sh  		Generate CMP’s	Pre-stack TOC

#! /bin/sh

set -x

# Purpose: To generate CMP values in the headers

#       Headers must already have the correct geometry values inserted for

#       the seismic experiment (See header_geom.sh for this)

#       We use the basic relation that

#

#                       CMP = (sx+gx)/2

#

#       where sx is the shot location, and gx is the receiver location.

#

#       We use suchw to calculate the CMP using offset and other key words as

#       input.

#            value(key1) = (a + b * value(key2) + c * value(key3)) / d

#       can be rewritten as:

#

#       If we choose the first CMP to be equal to ,say, 101

#       then a = 304

#       a = ( 101 (first CMP number) + 51 (absolute value of

#       half the longest offset on the first shot gather))/2

#       Because d=2 we have to double the size to get a and make

#       our first CMP=101.  You can choose other numbers to be the first CMP.

#

#            value(cdp)   =(304 +1 * value(sx)   + 1 * value(gx)  ) / 2

# Date: Nov. 18 2005

# Juan Lorenzo

# DATA/$file DIRECTORY

DATA=/data2/gom/Livingston_raw/seismics/data/SU/102205

suchw <$DATA/1001_38_geom.su \

key1=cdp        \

key2=sx         \

key3=gx         \

a=304           \

b=1             \

c=1             \

d=2             \

>$DATA/1001_38_CMP.su

study_NMO_STACK.sh	->NMO or STACK	Pre-stack TOC

#! /bin/sh

#HOME DIRECTORY

DATA=/home/juan/102205

WIDTH=210

HEIGHT=700

WIDTHOFF1=0

WIDTHOFF2=270

WIDTHOFF3=450

WIDTHOFF4=690

HEIGHTOFF=0

file1=1001_38_CMP.su

CMP_file=1001_38_CMP_sort.su

# The first thing to do when carrying out NMO is to

#collect your data as a function of CMP's

# Let's do that now by running susort.

#

# As the selfdoc says:

# Use the following to sort traces by cdp gather and within each gather

# by offset with both sorts in ascending order:

#Let's shorten the data set if you have not done so already

suwind <$DATA/$file1 tmin=0 tmax=0.6 >short_version

# sort the data into CMP gathers

susort <short_version >$DATA/$CMP_file cdp offset

# move out the data with a given constant velocity

vnmo=1500

#sunmo <$DATA/$CMP_file par=stkvel.p            \

susort <short_version cdp offset                \

|sunmo <$DATA/$CMP_file vnmo=$vnmo              \

| sugain agc=1 wagc=0.1                         \

| sustack key=cdp                               \

| suximage perc=$perc                           \

title="102205 vNMO=$vnmo"               \

windowtitle=""                          \

label1="Time (sec)" label2="Trace Number"       \

wbox=${WIDTH}                           \

hbox=${HEIGHT}                          \

xbox=${WIDTHOFF1}                       \

ybox=${HEIGHTOFF}                       \

clip=1                                  \

&

#####################################################

# Now onward to stacking!

sunmo < short_version                                 \

vnmo=$vnmo                                      \

> nmodata                                       \

susort < nmodata                       \

| sustack key=cdp                                      \

| sufilter f=80,120,400,600                            \

| sugain   agc=1 wagc=0.1                              \

| suximage                                             \

title="102205 vNMO=brute stack"                \

windowtitle=""                                 \

label1="Time (sec)" label2="Trace Number"      \

wbox=${WIDTH}                                  \

hbox=${HEIGHT}                                 \

xbox=${WIDTHOFF1}                              \

ybox=${HEIGHTOFF}                              \

clip=1                                         \

&