- ACOUSTIC WAVE EQUATION

- Introduction

- As we try to visualize the earth seismically we make certain physical simplifications that make it easier to make and explain our observations.  Liner (2004, p.33) considers three types of simplified descriptions for the earth materials: fluid, porous and solid.

- Fluids, e.g. gases and liquids characteristically can deform only by compression and decompression.  Only acoustic, or sound, or P-wave or compressional body waves are the natural wave vibrations that can propagate through fluid earth materials.

- In a fluid, seismic data can be collected by hydrophones [Q]in the form of pressure measurements.  The pressure field is a scalar field, which is simpler to deal with mathematically, than a tensor field.

- From the following mathematical derivations we can reach several accurate concepts to help us visualize particle strain during acoustic wave transmission in a fluid.

- First, within a fluid, and at a given point, particle motion increases as the pressure gradient increases.

- Second, the larger the particle density the slower the particle acceleration. Note that an unweathered piece of granite is denser than a piece of slate from a blackboard and so based ON ONLY the property of its density particle acceleration will be smaller in granite than in slate. [Q]

- Third, (not shown mathematically below, YET.. work in progress) the more incompressible the fluid the faster the particle motion.  A fast particle motion is related with a fast transmission of mechanical vibration through the fluid.  If we use the ball and spring model this implies that as we substitute stiffer and stiffer springs in the picture particle motion is faster and the vibration or wave is able to cross between balls at a faster rate (“speed of sound”).

- Mathematical Derivation

- We saw in the section on tensors that

Ti=niσij

The total force F=(F1,F2,F3)  exerted by the medium on to the volume Vthrough the small surface given area is

Fi=σijnjdA     where    σijnj is force per unit area (pressure)	(1)

For example in a fluid:

σij=−Pδij 			(2)

wherePis pressure and where compression is by convention negative.

- Expression (2) can also be expressed as

σ11000σ22000σ33=−P000P000Pδ11000δ22000δ33−P000P000P100010001

Note that we can view the Kroneker delata as a second order tensor where there are NO off-main-diagonal components because there is no shear, i.e.

σij=σii

- So now combining (1) and (2) we have

Fi=−PδijnjdA−PnidA

that is, in vectorial notation, F=−PndA.

If there is no gradient in the pressure there is no net force acting on it.  For example a neutrally buoyant sphere will neither rise nor fall  immersed in a fluid.

# On the bulk modulus and Lamé’s parameters

- Previously, in dealing with the elastic wave equation we saw that Hooke’s law for the case of an isotropic, heterogeneous medium took on the form

σij=λδijekk+2μeij

and the scalar expressions that related the strain field to the gradient of the displacement field or the dilatation were

ekk=uk,k=∇⋅u=ΔVV

For a hyrdrostatic pressure field where σ11=σ22=σ33=−Pwe can rephrase Hooke’s Law. substituting (2) to obtain :

−Pδij=λδijekk+2μeij	(3)

We contract the indices, making i=jin order to consider only the non-zero contributions to the sum:

−Pδii=λδiiekk+2μeii−P⋅3=λ⋅3⋅ekk+2μekk−P⋅3=λ⋅3+2μekk  (Note that ekk≡eii)

By replacing terms with other equivalent expressions, noted immediately above,

−P⋅3=λ⋅3+2μΔVVP=−λ⋅3+2μ3ΔVVP=−λ+23μΔVV

−PVΔV=λ+23μ

In this form, we can show that the bulk modulus (κ=−ΔPΔVV) can be expressed in terms of Lamé’s parameters:

- κ=λ+23μ

In the acoustic case, we have that μ=0and κ=λ, so that equation (3) can  be re-expressed as

=κ∇⋅u=κΔVV		(4)

from which we see that the divergence field is proportional to the pressure.

Particle acceleration and its relation to density the pressure field and wave velocity

We can predict the different parameters in the equation of motion:

ρu=σij,j−Pδij,j−P,jδij+Pδij,j−P,jδij=−P,iδii−P,i≡−∂P∂xi  (non-zero components only exist for j=i)

In vectorial notation we can also express this as

ρu=−∇Pu=−∇Pρ			(5)

In this form the equation of motion tells us that the particle acceleration in a body increases with larger gradients in the pressure field but decreases as the material becomes denser and requires more energy to move.

We can also estimate how pressure changes in space can affect the particle acceleration, by taking the divergence of the above expression.

−∇⋅∇P=∇⋅ρu−∇2P=ρ∇⋅u+u⋅∇ρρ∂2∂t2∇⋅u+u⋅∇ρ